Lab 3 Notes

Prescale

CLKEVOUT Notes

Part A

4 MHZ (40 ns/div)

4 MHZ (20 ns/div)

32 MHZ

32 MHZ

PART B

CTRLA controls how fast CNT is counted up. Then CNT is compared to PER

EBI

Part A Code

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

.equ stack_init=0x3FFF   ;initialize stack pointer

MAIN:

ldi YL, low(stack_init)    ;Load 0xFF to YL

out CPU_SPL, YL			   ;transfer to CPU_SPL

ldi YL, high(stack_init)   ;Load 0x3F to YH

out CPU_SPH, YL			   ;transfer to CPU_SPH

ldi r17, 0b00010100  ;divide by 8 to change from 32 MHZ to 4 MHZ in the subroutine. 0x14

ldi r16, 0b10000000   ;load value into r16. configure pin 7 as output

sts PORTC_DIRSET, r16 ;configure pin 7 as output

rcall CLK

ldi r16, 0b00001001   ;output CLKPER 4 on PORT C pin 7. pin 7 is the default. 0000=not used, 10=output CLKPER4, 01=PORTC

sts PORTCFG_CLKEVOUT, r16    ;output to PORTCFG_CLKEVOUT

DONE:

rjmp DONE         ;infinite loop to end the program

CLK:

push r16              ;push r16

ldi r16, 0b00000010   ;bit 1 is the 32Mhz oscillator

sts OSC_CTRL, r16     ;store r16 into the OSC_CTRL

NSTABLE:

lds r16, OSC_STATUS     ;load oscillator status into r16

bst r16, 1              ;check if 32Mhz oscillator is stable

brts STABLE             ;branch if stable

brtc NSTABLE            ;loop again if non-stable

STABLE:

ldi r16, 0xD8   ;writing IOREG to r16

sts CPU_CCP, r16 ;write IOREG to CPU_CCP to enable change

ldi r16, 0b00000001  ;write this to r16. corresponds to 32Mhz oscillator

sts CLK_CTRL, r16    ;select the 32Mhz oscillator

ldi r16, 0xD8    ;writing IOREG for prescaler

sts CPU_CCP, r16 ;for prescaler

sts CLK_PSCTRL, r17  ;r17 will be initialized outside the subroutine for prescale. 32/8=4MHZ

pop r16          ;pop r16

ret              ;return to main routine

Part B

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

MAIN:

ldi r17, 0xFF               ;load low byte

sts TCC0_PER, r17           ;0X826. should

ldi r17, 0x00              ;load high byte

sts TCC0_PER+1, r17         ;0x827

ldi r17, 0b00000111       ;prescaler CLK/1024

sts TCC0_CTRLA, r17       ;CTRLA controls the count (CNT)

ldi r17, 0xFF             ;set as output

sts PORTC_DIRSET, r17     ;set as output

REPEAT:

lds r17, TCC0_CNT

sts PORTC_OUT, r17

rjmp REPEAT

Part C

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

.set IN_PORT = 0x200000         ;beginning at end of the 32K SRAM. base address

;.set IPORT_END = 0x207FFF

MAIN:

ldi r16, 0x01   ; 4 bit data bus, data multiplexed with address byte 0 and 1, 3 port

sts EBI_CTRL, r16   ;configure mode to be 3 port

ldi r16, 0b00011101   ;set for 32K SRAM chip select

sts EBI_CS0_CTRLA, r16   ;set for 32K SRAM

ldi r16, 0b00010111   ;hex 0x17, enable CS0, RE, WE, ALE1

sts PORTH_DIRSET, r16 ; set CS0, RE, WE, ALE1 as output

ldi r16, 0b00010011     ;bit 0=we, bit 1=re, bit 4= CS0

sts PORTH_OUTSET, r16 ;set false value to CS0, WE, RE

ldi r16, 0b00001000   ;bit 3= ALE1

sts PORTH_OUTCLR, r16 ;set false value to ALE

ldi r16, 0xFF

sts PORTJ_DIRSET, r16

ldi r16, 0xFF

sts PORTK_DIRSET, r16

ldi ZL, low(EBI_CS0_BASEADDR)        ;Y pointer point to base address

ldi ZH, high(EBI_CS0_BASEADDR)

;ldi r16, byte3(EBI_CS0_BASEADDR)

;sts CPU_RAMPY, r16

ldi r16, byte2(IN_PORT)              ;transfer middle byte of base address to lower byte of base address

st Z+, r16

ldi r16, byte3(IN_PORT)              ;transfer high byte of base address to upper byte of base address

st Z, r16

ldi ZL, byte3(In_PORT)

out CPU_RAMPZ, ZL

ldi ZL, low(IN_PORT)

ldi ZH, byte2(IN_PORT)

REPEAT:

ldi r16, 0xA5

st Z, r16

ld r17, Z

rjmp REPEAT

Part C pins

0-2  = A2:0

3-10  =D7:0

11= ALE

12= CS0

13= WE

14=RE

What is this?

Part C LSA screenshot

Screenshots are all basically the same thing

Change per, calculate time it takes to do one second

Code

/* Lab 3 Part A

Name: Pengzhao Zhu

Section#: 112D

TA Name: Chris Crary

Description: This Program configures the XMEGA clock to run at 4MHZ or 32MHZ (depending on situation)

*/

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

.equ stack_init=0x3FFF   ;initialize stack pointer

MAIN:

ldi YL, low(stack_init)    ;Load 0xFF to YL

out CPU_SPL, YL			   ;transfer to CPU_SPL

ldi YL, high(stack_init)   ;Load 0x3F to YH

out CPU_SPH, YL			   ;transfer to CPU_SPH

ldi r17, 0b00010100  ;divide by 8 to change from 32 MHZ to 4 MHZ in the subroutine. 0x14

ldi r16, 0b10000000   ;load value into r16. configure pin 7 as output

sts PORTC_DIRSET, r16 ;configure pin 7 as output

rcall CLK

ldi r16, 0b00001001   ;output CLKPER 4 on PORT C pin 7. pin 7 is the default. 0000=not used, 10=output CLKPER4, 01=PORTC

sts PORTCFG_CLKEVOUT, r16    ;output to PORTCFG_CLKEVOUT

DONE:

rjmp DONE         ;infinite loop to end the program

CLK:

push r16              ;push r16

ldi r16, 0b00000010   ;bit 1 is the 32Mhz oscillator

sts OSC_CTRL, r16     ;store r16 into the OSC_CTRL

NSTABLE:

lds r16, OSC_STATUS     ;load oscillator status into r16

bst r16, 1              ;check if 32Mhz oscillator is stable

brts STABLE             ;branch if stable

brtc NSTABLE            ;loop again if non-stable

STABLE:

ldi r16, 0xD8   ;writing IOREG to r16

sts CPU_CCP, r16 ;write IOREG to CPU_CCP to enable change

ldi r16, 0b00000001  ;write this to r16. corresponds to 32Mhz oscillator

sts CLK_CTRL, r16    ;select the 32Mhz oscillator

ldi r16, 0xD8    ;writing IOREG for prescaler

sts CPU_CCP, r16 ;for prescaler

sts CLK_PSCTRL, r17  ;r17 will be initialized outside the subroutine for prescale. 32/8=4MHZ

pop r16          ;pop r16

ret              ;return to main routine

Part B

/* Lab 3 Part B

Name: Pengzhao Zhu

Section#: 112D

TA Name: Chris Crary

Description: This Program ulitizes the Timer system to allow the CNT register to count to 255. The CNT will increment one time

every 1024 clock cycles

*/

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

MAIN:

ldi r17, 0xFF               ;load low byte

sts TCC0_PER, r17           ;0X826. should

ldi r17, 0x00              ;load high byte

sts TCC0_PER+1, r17         ;0x827

ldi r17, 0b00000111       ;prescaler CLK/1024

sts TCC0_CTRLA, r17       ;CTRLA controls the count (CNT)

ldi r17, 0xFF             ;set as output

sts PORTC_DIRSET, r17     ;set as output

REPEAT:

lds r17, TCC0_CNT         ;load value from TCC0_CNT to r17

sts PORTC_OUT, r17        ;output value at r17 to PORTC

rjmp REPEAT               ;REPEAT

Part C

/* Lab 3 Part C

Name: Pengzhao Zhu

Section#: 112D

TA Name: Chris Crary

Description: This Program configures the 3-Port EBI system on the XMEGA. It will then write 0xA5 to all memory addresses

of the external 32K SRAM starting at address 0x200000.

*/

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

.set IN_PORT = 0x200000         ;beginning at end of the 32K SRAM. base address

;.set IPORT_END = 0x207FFF

MAIN:

rcall EBI                  ;call EBI subroutine

STARTOVER:

ldi ZL, byte3(In_PORT)        ;load 0x20 into ZL

out CPU_RAMPZ, ZL             ;ZL value into RAMPZ

ldi ZL, low(IN_PORT)          ;load 0x00 into ZL

ldi ZH, byte2(IN_PORT)        ;load 0x00 into ZH

REPEAT:

ldi r16, 0xA5                 ;load 0xA5 into r16

st Z, r16                     ;r16 value into value pointed by Z pointer

cpi ZH, 0x7F                  ;first check for if 32K limit is reached

breq CHECK                    ;if equal, branch to do second check

LOAD:

ld r17, Z+                    ;load value to Z pointer address to r17. post increment

rjmp REPEAT                   ;jump to REPEAT

CHECK:

cpi ZL, 0xFF                   ;second check for if 32K limit is reached

breq STARTOVER                 ; if limit is reached. start over from 0x200000

brne LOAD                      ;if not, branch to LOAD

EBI:   ;takes in IN_PORT, IN_PORT = 24 bit address for 32 K SRAM

push r16                       ;push r16

push ZL                        ;push ZL

push ZH                        ;push ZH

ldi r16, 0x01   ; 4 bit data bus, data multiplexed with address byte 0 and 1, 3 port

sts EBI_CTRL, r16   ;configure mode to be 3 port

ldi r16, 0b00011101   ;set for 32K SRAM chip select

sts EBI_CS0_CTRLA, r16   ;set for 32K SRAM

ldi r16, 0b00010111   ;hex 0x17, enable CS0, RE, WE, ALE1

sts PORTH_DIRSET, r16 ; set CS0, RE, WE, ALE1 as output

ldi r16, 0b00010011     ;bit 0=we, bit 1=re, bit 4= CS0

sts PORTH_OUTSET, r16 ;set false value to CS0, WE, RE

ldi r16, 0b00000100   ;bit 2= ALE1

sts PORTH_OUTCLR, r16 ;set false value to ALE

ldi r16, 0xFF                    ;0xFF to r16

sts PORTJ_DIRSET, r16            ;set PORTJ to be output

ldi r16, 0xFF                    ;0xFF to r16. unneccesary, but I am still including it

sts PORTK_DIRSET, r16            ;set PORTK to be output

ldi ZL, low(EBI_CS0_BASEADDR)        ;ZL pointer point to low byte of EBI_CS0_BASEADDR

ldi ZH, high(EBI_CS0_BASEADDR)       ;ZH pointer point to high byte of EBI_CS1_BASEADDR

ldi r16, byte2(IN_PORT)              ;transfer middle byte of base address to lower byte of base address

st Z+, r16                           ;r16 value to address pointed to Z pointer. post increment Y pointer

ldi r16, byte3(IN_PORT)              ;transfer high byte of base address to upper byte of base address

st Z, r16                            ;r16 value to address pointed to Z pointer

pop ZH                               ;pop ZH

pop ZL                               ;pop ZL

pop r16                              ;pop r16

ret                                  ;return from subroutine

Part D

/* Lab 3 Part D

Name: Pengzhao Zhu

Section#: 112D

TA Name: Chris Crary

Description: This Program stores data read from the DIP switches to sequential memory locations every one second.

It will then read it back, and store the value to the LED bank.

*/

.include "ATxmega128A1Udef.inc"        ;include the file

.list                                  ;list it

.org 0x0000                            ;start our program here

rjmp MAIN                              ;jump to main

.equ stack_init=0x3FFF   ;initialize stack pointer

.equ IN_PORT=0x300000

MAIN:

ldi YL, low(stack_init)    ;Load 0xFF to YL

out CPU_SPL, YL			   ;transfer to CPU_SPL

ldi YL, high(stack_init)   ;Load 0x3F to YH

out CPU_SPH, YL			   ;transfer to CPU_SPH

ldi r17, 0x00  ;set up 32MHZ clock in

rcall CLK      ;subroutine to set up 32Mhz clock

rcall EBI      ;subroutine to set up EBI of 32K and base address of 0x300000

ldi r16, 0xFF

sts PORTA_DIRCLR, r16    ;set Port A to be input

sts PORTC_DIRSET, r16    ;set Port C to be output

sts PORTC_OUTSET, r16    ;turn off the LED for now

STARTOVER:

ldi ZL, byte3(In_PORT)    ;load highest byte of 0x300000

out CPU_RAMPZ, ZL         ;RAMPZ point to 30

ldi ZL, byte1(IN_PORT)    ;ZL point to 00

ldi ZH, byte2(IN_PORT)    ;ZH point to 00

REPEAT:

lds r16, PORTA_IN         ;take in value from input switches

cpi ZH, 0x7F              ;check if middle byte is 0x7F to see if we have reach the limit of the external SRAM

breq CHECK                ;if equal, branch to check again

LOAD:

st Z, r16          ; write to external memory

ld r18, Z+         ;read it back from external memory

rcall TIMER        ;call timer

sts PORTC_OUT, r18 ;output to LED

rjmp REPEAT        ;repeat

CHECK:

cpi ZL, 0xFF       ;check if the lower byte is 0xFF to see if we have reach the limit of the 32K external SRAM

breq STARTOVER     ;if we have. start from 0x300000 again

brne LOAD          ; otherwise, branch to load

; the rest are just subroutines

CLK:   ;take in a r17 value for prescaler. 32MHZ = 0x00 for prescale

push r16              ;push r16

ldi r16, 0b00000010   ;bit 1 is the 32Mhz oscillator

sts OSC_CTRL, r16     ;store r16 into the OSC_CTRL

NSTABLE:

lds r16, OSC_STATUS     ;load oscillator status into r16

bst r16, 1              ;check if 32Mhz oscillator is stable

brts STABLE             ;branch if stable

brtc NSTABLE            ;loop again if non-stable

STABLE:

ldi r16, 0xD8   ;writing IOREG to r16

sts CPU_CCP, r16 ;write IOREG to CPU_CCP to enable change

ldi r16, 0b00000001  ;write this to r16. corresponds to 32Mhz oscillator

sts CLK_CTRL, r16    ;select the 32Mhz oscillator

ldi r16, 0xD8    ;writing IOREG for prescaler

sts CPU_CCP, r16 ;for prescaler

sts CLK_PSCTRL, r17  ;r17 will be initialized outside the subroutine for prescale. 32/8=4MHZ

pop r16          ;pop r16

ret              ;return to main routine

EBI:   ;takes in IN_PORT, IN_PORT = 24 bit address for 32 K SRAM

push r16

push ZL

push ZH

ldi r16, 0x01   ; 4 bit data bus, data multiplexed with address byte 0 and 1, 3 port

sts EBI_CTRL, r16   ;configure mode to be 3 port

ldi r16, 0b00011101   ;set for 32K SRAM chip select

sts EBI_CS0_CTRLA, r16   ;set for 32K SRAM

ldi r16, 0b00010111   ;hex 0x17, enable CS0, RE, WE, ALE1

sts PORTH_DIRSET, r16 ; set CS0, RE, WE, ALE1 as output

ldi r16, 0b00010011     ;bit 0=we, bit 1=re, bit 4= CS0

sts PORTH_OUTSET, r16 ;set false value to CS0, WE, RE

ldi r16, 0b00000100   ;bit 2= ALE1

sts PORTH_OUTCLR, r16 ;set false value to ALE

ldi r16, 0xFF

sts PORTJ_DIRSET, r16            ;set port J to be output

sts PORTK_DIRSET, r16            ;set port K to be output

ldi ZL, low(EBI_CS0_BASEADDR)        ;ZL pointer point to low byte of EBI_CS0_BASEADDR

ldi ZH, high(EBI_CS0_BASEADDR)       ;ZH pointer point to high byte of EBI_CS1_BASEADDR

ldi r16, byte2(IN_PORT)              ;transfer middle byte of base address to lower byte of base address

st Z+, r16                           ;r16 value to Z pointer address. post increment Z pointer

ldi r16, byte3(IN_PORT)              ;transfer high byte of base address to upper byte of base address

st Z, r16                            ;r16 value to Z pointer address.

pop ZH                               ;pop ZH

pop ZL                               ;pop ZL

pop r16                              ;pop r16

ret                                  ;return from subroutine

TIMER:   ;delay for 1 second

push r17				    ;push r17

ldi r17, 0x12               ;load low byte of 0x7A12

sts TCC0_PER, r17           ;transfer to TCC0_PER

ldi r17, 0x7A              ;load high byte of 0x7A12

sts TCC0_PER+1, r17         ;transfer to TCC0_PER+1

ldi r17, 0b00000111       ;prescaler CLK/1024

sts TCC0_CTRLA, r17       ;CTRLA controls the count (CNT)

NOTSET:

nop                        ;delay

lds r17, TCC0_INTFLAGS     ;check if the flag is set

bst r17, 0                 ;check the zero bite

brts RETURN                ;if set, branch to return

brtc NOTSET                ;if not, branch to NOTSET and continue

RETURN:

ldi r17, 0x01            ;to clear the flag

sts TCC0_INTFLAGS, r17   ;clears the flag

pop r17                  ;pop r17

ret                      ;return from subroutine